PURPOSE:

To provide a response guide in case of a magnet quench in the Yale MRRC

SCOPE:

This procedure applies to the Yale MRRC imaging floor.

RESPONSIBILITY:

Personnel involved in MR imaging research.

PROCEDURE:

- Do not panic.

- Understand that a quench does not mean that the magnet has lost all of its magnetism.

- Make sure no metal objects enter the scan room.

- Use the intercom to tell subject to stay calm and that someone will be in immediately to help him or her.

- Prop open the door between the magnet scanner room and the console.

- Enter the room, help the subject off the scanner table and exit the scan room.

- Evacuate all personnel from the area.

- Proceed to your department’s meeting place, which is the CORNER of CONGRESS and CEDAR STREETS.

- Call Yale Security: 785-5555.

- Call Local Building Operations Manager for MRRC, Terry Nixon, at 785-6198 or 203-988-2294.

- Do not re-enter the scanning area until emergency personnel declare that it is safe to do so.