Packaging Extracts

Media: 	NZY Broth

10.5 g NZY

500 ml dH20

1.8 ml 1M NaOH

Autoclave!

Solutions (make fresh day of use, filter sterilize all, and keep on ice)

# Sucrose Solution

1 g sucrose

500 l 1M TrisCl, pH 8.0

9.5 ml ddH20

## Lysozyme Solution

0.020 g lysozyme

100 l 1M TrisCl, pH 8.0

9.9 ml ddH20

## Sonication Buffer

400 l 1M TrisCl, pH 8.0

40 l 0.5M EDTA

7 l BME

up to 20 ml ddH2O

## Packaging Buffer

15 l 1M TrisCl, pH 8.0

0.032 g spermidine

0.020 g putrescine

50 l 1M MgCl2

5.25 l BME

750 l 0.1M ATP. pH 7.0

up to 2.5 ml ddH2O

### Packaging Extracts: Freeze Thaw Day 1 and 2

#### 2 Days Before:	Streak 2 fresh LB plates with BHB2688

Grow overnight at 30C and 42C

(42 is only to check the strain for contamination)

1 Day Before:	Prepare 3L NZY Broth in 6 2L flasks

(plus additional NZY Broth for overnight culture)

Inoculate 200 ml NZY with a colony of BHB2688

Grow overnight: shaking incubator, 30C (~15-16 hours)

Day 1:	Inoculate 6 flasks with 25 ml each of overnight culture

Grow in 32C shaking incubator until OD600=0.6 (2-3 hrs)

Transfer flasks to 60C shaking waterbath (I do 3 at a time)

Read internal temp, once it reaches 45C transfer flasks to 45C

waterbath for 15 minutes, shaking periodically

Return flasks to shaking incubator now set at 38-39C

(do not exceed 40C)

Shake for 2-3 hours

Near end of time remove 2 ml of culture and add 1 ml each into

2 tubes, add 5-6 drops chloroform to one to see if it clears when kept at 37C for 2-3 minutes, if clears

Spin all cultures in large bottles, 4800 RPM, 10 min, 4C

Drain off supernatant, wipe inside of bottles with Kimwipes

(do not disturb pellets!)

Resuspend all pellets in a total of 6 ml sucrose solution

Dispense 500 l aliquots (using wide bore tip) into 1.5 ml tubes

with 25 l lysozyme solution, mixing as you go

Quick freeze with liquid Nitrogen, store at -80C

Day 2:	Thaw all tubes on ice for at least 1 hour

Add 25 l packaging buffer to each tube (keeping on ice!)

Use glass spooler and wide bore tip to combine “goop” into 2

oak ridge tubes

Spin in ultracentrifuge 45,000 RPM, 3 hrs, 4C (TY65 rotor)

Carefully transfer supernatant from pellet with Pasteur

pipette into chilled 15 ml conical tube

Aliquot 80 l per tube, quick freeze with liquid Nitrogen

Store at -80C

### Packaging Extracts: Sonicate Extracts

#### 2 Days Before:	Streak 2 fresh LB plates with NM759

Grow overnight at 30C and 42C

(42 is only to check the strain for contamination)

1 Day Before:	Prepare 1L NZY Broth in 2 2L flasks

(plus additional NZY Broth for overnight culture)

Inoculate 75 ml NZY with a colony of BHB2688

Grow overnight: shaking incubator, 30C (~15-16 hours)

Day 1:	Inoculate 2 flasks with 25 ml each of overnight culture

Grow in 32C shaking incubator until OD600=0.3 (2-3 hrs)

Transfer flasks to 60C shaking waterbath

Read internal temp, once it reaches 45C transfer flasks to 45C

waterbath for 15 minutes, shaking periodically

Return flasks to shaking incubator now set at 38-39C

(do not exceed 40C)

Shake for 2-3 hours

Near end of time remove 2 ml of culture and add 1 ml each into

2 tubes, add 5-6 drops chloroform to one to see if it clears when kept at 37C for 2-3 minutes, if clears

Spin all cultures in large bottles, 4800 RPM, 10 min, 4C

Drain off supernatant, wipe inside of bottles with Kimwipes

(do not disturb pellets!)

Resuspend all pellets in a total 7.2 sonication buffer and

transfer to 2 polypropylene tubes

Sonicate in 3 second bursts 10-14 times, keeping on ice 30

seconds between each time

Spin 10,000 RPM, 10 minutes, 4C

Transfer supernatant to chilled 15 ml conical tube

Add 1/2 volume (of supernatant) sonication buffer

Add 1/6 (original) volume packaging buffer

Aliquot 120 l per tube

Quick freeze with liquid Nitrogen, store at -80C